RumblePak – Format

RumblePaks are pretty limited in their behaviour, they have a single Register at Adress 0x0C00 for starting/stoping Rumble. Different strengths are accomplished by turning them on/off  in different amount of time.

The Behaviour  of a RumblePak is this:

Upper Range ( 0x0000 – 0x7FF )

Pak - Writes don’t have an effect.

Pak - Reads will return Blocks of 0x00

Test Block( 0x8000 - 0x80FF )

Read the MemPak.doc for generous purpose(IMHO) of this Block( or maybe Register ?)

Pak - Writes don’t have an effect, they just determine what value is sent back later.

Pak - Reads will return Blocks of 0x80 if the last Value written to that block was nonzero, and returns Blocks of 0x00 if the last Value was zero

Rumble Register ( 0xC000 )

Pak – Writes should either be a block of 32 0x01 Bytes or a block of 32 0x00 Bytes.

0x01 = start Rumble

0x00 = stop Rumble

Pak – Reads: not tested

© 2001 N-Rage ( go.to/nrage )